Below is the final, consolidated list of components you are using so far, based on everything you tested and confirmed.

# ✅ FINAL COMPONENT LIST (POWER + ACTUATION + CORE)

## 🔋 Power System

### 1️⃣   Lithium-ion Battery

- Type: Single-cell Li-ion

- Nominal Voltage: 3.7 V

- Capacity: 1100 mAh

- Purpose: Primary power source for the system

### 2️⃣   USB-C Charging Module

- IC: TP4056 (with protection)

- Input: 5 V (USB-C)

- Function: Safe charging of the 3.7 V Li-ion battery

- Features:

  - Over-charge protection

  - Over-discharge protection

  - Short-circuit protection

### 3️⃣   Voltage Regulator

- Part Number: HT7333

- Type: Low Dropout Regulator (LDO)

- Output Voltage: 3.3 V

- Max Output Current: ≈ 250 mA

- Package: TO-92 (through-hole)

- Status: ✅ Tested & working

- Purpose: Regulates battery voltage to stable 3.3 V for ESP32 and sensors

### 4️⃣   Decoupling & Bulk Capacitors

| Value | Type | Voltage Rating | Placement |
| --- | --- | --- | --- |
| 10 µF | Ceramic (X7R) | ≥10 V | HT7333 input |
| 22 µF | Ceramic (X7R) | ≥10 V | HT7333 output |
| 100 µF | Electrolytic | ≥6.3 V | Near ESP32 3V3 pin |
| (Optional) 0.1 µF | Ceramic | ≥10 V | Near ESP32 VCC |

Purpose:

- Voltage stability

- Suppression of transients

- Prevent ESP32 brownout during Wi-Fi bursts

## 🧠 Processing & Communication

### 5️⃣   Microcontroller

- Board: ESP32 Dev Module

- Supply Voltage: 3.3 V (via HT7333)

- Features Used:

  - Wi-Fi

  - GPIO

  - Sensor interfaces

- Status: ✅ Tested on battery + Wi-Fi (stable)

## 🌡️ Sensors (Tested & Working)

### 6️⃣   Heart Rate Sensor

- Model: MAX30105

- Interface: I²C

- Supply: 3.3 V

- Status: ✅ Tested & working

### 7️⃣   Temperature & Humidity Sensor

- Model: HTU21D

- Interface: I²C

- Supply: 3.3 V

- Status: ✅ Tested & working

## 🔔 Actuation (Next to test)

### 8️⃣   Vibration Motor

- Type: Coin or cylindrical vibration motor

- Supply: Battery voltage (3.7 V)

- Purpose: Haptic alert

### 9️⃣   N-Channel MOSFET

- Part Number: 2N7000

- Package: TO-92

- Gate Voltage: 3.3 V (ESP32 GPIO)

- Purpose: Low-side switching of vibration motor

- Status: ⚠️ Not yet tested (but electrically correct)

### 🔹 Protection Component (Mandatory with motor)

- Flyback Diode

  - Type: 1N4148 or 1N4007

  - Connected across motor

  - Purpose: Protect MOSFET from inductive voltage spikes

## 🔧 Passive & Miscellaneous Components

### 10️⃣   Resistors

- Gate resistor for MOSFET: 100 Ω – 220 Ω

- Gate pull-down resistor: 10 kΩ

### 11️⃣   Prototyping & Interconnect

- Perfboard / prototype PCB

- Male & female header pins

- Jumper wires

- On/off slide switch (optional, for battery isolation)

# 🏆 System Status Summary

| Subsystem | Status |
| --- | --- |
| Battery power | ✅ Working |
| HT7333 regulation | ✅ Tested & stable |
| ESP32 Wi-Fi | ✅ Stable |
| Sensors | ✅ Working |
| Charging circuit | ⚠️ Ready, not yet tested |
| Vibration motor | ⚠️ Pending test |

## 📘 Report-ready summary sentence

You can safely use something like this:

The system is powered by a 3.7 V, 1100 mAh lithium-ion battery charged via a TP4056 USB-C module. A low-dropout HT7333 regulator provides a stable 3.3 V supply to the ESP32 and sensors. Proper decoupling and bulk capacitors ensure reliable operation during Wi-Fi transmission. Actuation is achieved using a vibration motor driven through a logic-level N-channel MOSFET.